RED BRICK

APPLICATION & DETAILING GUIDE

A working companion for homeowners, architects and designers specifying brick

12 checklists and worksheets for planning, detailing and specifying red brick applications

How to use this guide

Work through the sections in order for a new project, or jump directly to the

checklist you need. Tick boxes, fill blanks, and keep this alongside your drawings.

# Contents

1. Brick Application Checklist

2. Location Selection Worksheet

3. Indoor vs Outdoor Application

4. Brick + Material Combination Matrix

5. Climate Considerations Worksheet

6. Brick Pattern Selection Guide

7. Screen / Jali Planning Checklist

8. Landscape Integration Checklist

9. Drainage and Water Checklist

10. Maintenance Planning Worksheet

11. Material Sample Comparison Sheet

12. Final Specification Checklist

# 1. Brick Application Checklist

Use this before any drawing is finalized. It forces a decision on role, not just placement — a brick element without a clear job is usually the first thing that gets redesigned later.

## For each proposed brick element, confirm:

☐  The specific location has been named (not "somewhere in the courtyard" but "south courtyard wall, 2.4m run")

☐  A single primary role has been assigned: flooring, path, edge, screen, jali, seating, feature wall, landscape element, or connector

☐  A secondary role, if any, has been identified (e.g. a seating wall that also retains a planter)

☐  The role has been checked against the space's actual use — a seating wall in a rarely-used corner is a wasted opportunity

☐  The element's relationship to at least one adjacent material has been decided

☐  Exposure to weather (sun, rain, splash-back) has been assessed for this specific location

☐  A maintenance expectation has been set and communicated to the client or household

☐  The element has been checked against the overall material palette so it doesn't compete with another feature

# 2. Location Selection Worksheet

Fill one row per space under consideration. This worksheet is deliberately blank — it's meant to be completed against your actual plan, not a generic house.

| Space / Location | Candidate Brick Role | Exposure Level | Priority (H/M/L) | Notes |
| --- | --- | --- | --- | --- |

Exposure level guide: rate each row Low (fully sheltered), Medium (occasional rain or splash), or High (direct sun and rain most of the year).

# 3. Indoor vs Outdoor Application

Brick behaves differently once it crosses the threshold. This isn't about whether brick "works" indoors or outdoors — it does, in both — but about which considerations shift priority.

## Indoor applications — priority checks

☐  Dust generation from a raw or lightly-sealed brick surface, particularly on floors

☐  Sealing or finishing method, if any, and its effect on colour and sheen

☐  Cleaning access — can the surface be reached and cleaned without special equipment

☐  Acoustic effect of a hard brick surface in a room that also needs sound absorption

☐  Compatibility with underfloor heating or cooling, where relevant

☐  Skirting, junction and threshold detailing where brick meets another indoor flooring material

## Outdoor applications — priority checks

☐  Slip resistance of the surface finish, especially where the area gets wet

☐  Freeze-thaw exposure in cooler climates, or thermal expansion in very hot ones

☐  UV and colour-fastness of any sealant used

☐  Drainage falls built into the paving base, not just the visible surface

☐  Plant growth, root intrusion and joint planting where brick meets soil

☐  Long-term weathering — will the brick be expected to age visibly, and is that acceptable

# 4. Brick + Material Combination Matrix

A quick reference for how brick tends to behave next to four common materials. Use this as a starting conversation with your designer, not a final specification — actual performance depends on the specific products and details used.

| Pairing | Visual Effect | Watch For |
| --- | --- | --- |
| Brick + Timber | Warmth reinforced by warmth; softens brick's hardness | Timber movement at the junction; keep timber clear of standing water |
| Brick + Concrete | Textural contrast; concrete reads cooler and more mass-like | Colour clash if concrete is left natural grey against warm red brick |
| Brick + Stone | Rhythm (brick) against weight (stone); strong hierarchy | Scale mismatch if stone units are too small relative to brick coursing |
| Brick + Metal | Sharp, precise edges against a textured, irregular surface | Metal expansion/contraction at fixed junctions; corrosion at damp contact points |
| Brick + Glass | Solid-vs-transparent contrast; can frame brick as a backdrop | Reflections can visually double the brick pattern in unintended ways |
| Brick + Greenery | Softens all of the above; brings colour and seasonal change | Root damage to joints; moisture retention against the brick face |

# 5. Climate Considerations Worksheet

Identify your climate zone, then work through the matching checklist. If your site sits between zones (e.g. a hot-dry region with a short monsoon), use both relevant checklists.

## Hot climate

☐  Wall orientation checked against sun path for each proposed brick wall

☐  Shading strategy identified for any large unshaded brick surface

☐  Ventilation path considered alongside, not instead of, thermal mass

☐  Realistic expectation set for how much cooling effect the brick will actually contribute

## Warm-humid climate

☐  Rain exposure mapped for each brick surface, including wind-driven rain

☐  Drying time considered — does the surface get enough sun/air to dry between rain events

☐  Algae or staining risk flagged for shaded, damp-prone surfaces

☐  Ventilation gaps sized to actually move air, not just look porous

## Hot-dry climate

☐  Diurnal temperature swing considered in relation to brick thermal mass

☐  Shading provided for surfaces with long, direct afternoon exposure

☐  Dust and wind-blown grit accounted for in surface and joint choice

Temperate / cool climate

☐  Overall thermal envelope strategy defined — brick is not treated as a stand-alone insulator

☐  Moisture management detailing checked, including damp-proofing where required

☐  Freeze-thaw resistance of the specific brick and mortar confirmed, where relevant

# 6. Brick Pattern Selection Guide

Match the pattern to the effect you actually want, not to what looks striking in a single photograph. This table pairs each pattern with the situations it tends to suit best.

| Pattern | Best Suited To | Effect |
| --- | --- | --- |
| Running bond | Long paths, large floors, calm feature walls | Directional, understated, easy to read |
| Herringbone | Courtyards, entrances, feature floors | Energetic, adds visual richness |
| Basket weave | Square courtyards, patios | Static, grid-like, formal |
| Soldier course border | Edges of any paved area | Frames a space, signals a boundary |
| Radial / fan | Focal points — tree bases, circular courtyards | Draws the eye to a centre point |
| Mixed-format | Thresholds, transition zones | Marks a change without a hard stop |

# 7. Screen / Jali Planning Checklist

Perforated brickwork is the application most often specified on aesthetics alone. This checklist pulls the performance variables back into the conversation before construction.

☐  Orientation of the screen relative to sun path has been checked, not assumed

☐  Open-area percentage has been calculated and compared against the privacy/light goals

☐  Offset distance between the screen and the wall or window behind it has been set

☐  Prevailing wind direction has been checked against the screen's orientation

☐  Structural support for the perforated assembly has been confirmed by an engineer where the screen is more than a low garden wall

☐  Pattern has been tested (sketch or model) for the shadow effect at more than one time of day

☐  Cleaning and access strategy for both faces of the screen has been considered

☐  Green buffer or planting behind/around the screen has been planned, if part of the design intent

# 8. Landscape Integration Checklist

Brick and planting fail most often at the joint between them, not within either material alone. Work through this before construction starts, not after the garden is planted.

☐  Soil depth confirmed at every planter wall, tree surround or garden edge

☐  Drainage path confirmed from planting bed away from brick paving or walls

☐  Root-intrusion risk assessed for any tree or large shrub near brick paving joints

☐  Irrigation or watering method confirmed not to pool against brick surfaces

☐  Joint planting (moss, groundcover) specified only where foot traffic and maintenance allow it

☐  Retaining function of any brick wall confirmed with appropriate structural backing, not decorative-only construction

# 9. Drainage and Water Checklist

This is the checklist most commonly skipped, and the one most responsible for long-term brick failures. Confirm each item for every brick application that sees rain, irrigation or splash.

☐  Falls (slope) built into the paving base, verified on the drawings, not just assumed at site

☐  Drainage outlets or channels positioned before, not after, paving is laid

☐  Damp-proof course specified where brickwork meets the ground

☐  Flashing detailed at any junction where brick meets a roof, parapet or projecting element

☐  Weep holes provided where cavity or veneer brick construction requires them

☐  Splash-back zone identified at the base of any wall near a downpipe or roof edge

☐  Water table or ground moisture conditions checked for below-grade brickwork

# 10. Maintenance Planning Worksheet

Set expectations before construction, not after the first monsoon. Fill this in per major brick element.

| Brick Element | Expected Inspection Frequency | Typical Maintenance Task | Who's Responsible |
| --- | --- | --- | --- |

Common maintenance tasks to consider: re-pointing degraded mortar joints, clearing planting joints of invasive growth, checking and clearing drainage outlets, re-sealing indoor floors on the manufacturer's recommended cycle, and removing algae or staining from shaded, damp-prone surfaces.

# 11. Material Sample Comparison Sheet

Bring physical samples to site before finalizing a specification. Photographs and screens shift colour more than most people expect.

| Sample | Brick Type / Source | Colour in Shade | Colour in Direct Sun | Texture Notes | Decision |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |

Tip: view every sample at the actual time of day and orientation it will be installed in — a brick that looks warm in morning east light can look flat and grey on a shaded south wall.

# 12. Final Specification Checklist

The last check before brickwork is ordered and construction begins. If any item here is unresolved, resolve it before proceeding — changes after ordering are expensive and slow.

☐  Brick type, size, colour range and source confirmed and sampled on site

☐  Mortar colour and joint width confirmed against a sample panel, not a swatch alone

☐  Pattern and coursing confirmed for every distinct brick element

☐  Bond type confirmed for structural walls where relevant

☐  Drainage, damp-proofing and flashing details signed off by the person responsible for construction

☐  Junctions with every adjacent material (timber, stone, concrete, metal, glass, planting) detailed on the drawings

☐  Structural engineering sign-off obtained for any screen, jali or freestanding element beyond a low garden wall

☐  Maintenance plan communicated to the client or household in writing

☐  Sample panel built on site and approved before full construction proceeds

A material becomes more powerful when it performs more than one architectural role.